# FlexMatrix Command Line Instruction

## Overview

The FlexMtrix command line programmer CMLProgrammer can enable a production automation process. You may call it from command line from BAT file or from your application.

## Execute a Command

Command Line Format:

FlexMatrix CMLProgrammer.exe	/cmd	[parameter]

Where cmd can be any command listed in Command List section.

Examples: Program FormatABTpl-SK5126.bin to chip

>"FlexMatrix Programmer" /P FormatABTpl-SK5126.bin

The result of each execution is stored in a text file “Result_Report_File.txt”. At the same time, the execution result will be appended to the log file “Results_Log_File.txt”.

In the result file, the return values are listed line by line. Each line follows the format of “ValueName: Value”. The key value is “ReturnValue”. When the execution succeeds, the ReturnValue is 0; otherwise it’s a negative value.

## Command List

### Command “P” or “p” – Program FlexMatrix Bin File to Chip

Format: FlexMatrix CMLProgrammer.exe	/P	[FlexMatrix Bin FileName]

This command programs the specified FlexMatrix bin file to the attached USB chip, and verifies the execution result.

### Command “V” or “v” – Verify FlexMatrix Bin File with the Data in the Chip

Format: FlexMatrix CMLProgrammer.exe	/V	[FlexMatrix Bin FileName]

This command verifies the specified FlexMatrix bin file with the data in the attached USB chip.

### Command “R” or “r” – Read from Chip to a Bin File

Format: FlexMatrix CMLProgrammer.exe	/R	[SaveTo FileName]

This command reads the data from the attached UBB chip to the specified file.

### Command “L” or “l” – Get Checksum of FlexMatrix Bin File by Loading

Format: FlexMatrix CMLProgrammer.exe	/L	[FlexMatrix Bin FileName]

This command load the specified FlexMatrix bin file into memory and calculates the checksum.

# Appendices

## Appendix A – CML Programmer ReturnValue List

| ReturnValue | Description |
| --- | --- |
| 0 | Successful |
| -1001 | Device is not found |
| -1002 | ProgramData block size is not matched |
| -1003 | VerifyData block size is not matched |
| -1004 | File doesn't exist |
| -1005 | File name is missing |
| -1006 | Save to file failed |

## Appendix B – CML Programmer Result File and Log File Example

The Result_Report_File.txt will be the content between two separation lines.

--------------------------------------------------------------------------------------------

CMLProgrammerVersion: 1.32.1

DateTime: 8/21/2018 5:14:52 PM

Operation: Program and Verify.

FileName: FormatABTpl-SK5126.bin

Device is found.

PartNo: 5126

Version: 1.13

Revision: 1

Write Completed.

Read Completed.

Verification --- PASSED (1)

ReturnValue: 0 (Successful)

LoadedBufferChecksum: 0x1DFF0

LoadedBufferSize: 2560

VerifyBufferSize: 2560

--------------------------------------------------------------------------------------------

CMLProgrammerVersion: 1.32.1

DateTime: 8/21/2018 5:15:14 PM

Operation: Verify.

FileName: SK5126_Test.bin

Read Completed.

Device is found.

ReturnValue: -1003 (VerifyData contents are not matched)

LoadedBufferChecksum = 0x1E02C vs TargetBufferChecksum = 0x1DFF0

--------------------------------------------------------------------------------------------

CMLProgrammerVersion: 1.32.1

DateTime: 8/21/2018 5:15:30 PM

Operation: Verify.

FileName: FormatABTpl-SK5126.bin

Read Completed.

ReturnValue: 0 (Successful)

LoadedBufferChecksum: 0x1DFF0

LoadedBufferSize: 2560

VerifyBufferSize: 2560

--------------------------------------------------------------------------------------------

CMLProgrammerVersion: 1.32.1

DateTime: 8/21/2018 5:18:39 PM

Operation: Verify

FileName: FormatABTpl-SK5126232.bin

There no such file name!

ReturnValue: -1004 (File doesn't exist)

--------------------------------------------------------------------------------------------

CMLProgrammerVersion: 1.32.1

DateTime: 8/21/2018 5:19:22 PM

Operation: Verify

ReturnValue: -1005 (File name is missing)

--------------------------------------------------------------------------------------------

CMLProgrammerVersion: 1.32.1

DateTime: 8/21/2018 5:19:48 PM

Operation: Verify

FileName: FormatABTpl-SK5126.bin

ReturnValue: -1001 (Device is not found)

LoadedBufferChecksum: 0x1DFF0